CVT Opcodes & Rounding Modes

ARM  ·  RISC-V  ·  x86/Intel  ·  IBM  ·  Ternary ISA

With Ternary-Native Trit-Zone Rounding Proposal

# 1.  The Conversion Problem

Every CPU must answer the same question when converting between numeric formats: what do you do with precision that cannot be represented in the destination type? A 32-bit float converted to a 16-bit integer loses its fractional part. A wide floating-point value narrowed to fewer mantissa bits loses its least-significant precision. The CPU must make a decision: round up, round down, round toward zero, or round toward the nearest representable value. Different companies have made different choices, and those choices have real consequences for numerical software.

IEEE 754-2008 defines five standard rounding modes that most modern CPUs implement. The problem is that two of those modes — round-to-nearest-even (the default) and round-to-nearest-max-magnitude — require detecting the exact midpoint case, which adds hardware complexity and causes the infamous "banker's rounding" surprises in software that assumes normal rounding. Binary's midpoint problem does not exist in balanced ternary — as the final section of this document shows.

| IEEE Mode | Name | Behavior at 0.5 | Hardware cost |
| --- | --- | --- | --- |
| RNE | Round to Nearest Even | Rounds to even digit — 0.5 → 0, 1.5 → 2 | High — must detect exact midpoint |
| RTZ | Round Toward Zero | Truncate — always toward zero | Low — just drop bits |
| RDN | Round Down (floor) | Always toward -∞ | Low — compare sign |
| RUP | Round Up (ceiling) | Always toward +∞ | Low — compare sign |
| RMM | Round to Nearest Max Magnitude | 0.5 always rounds away from zero | High — must detect exact midpoint |

# 2.  x86 / Intel

## 2.1  SSE / AVX CVT Family

Intel's SSE/AVX extension set introduced a family of CVTSS2SI, CVTSD2SI, CVTTSS2SI, and related opcodes. The double-T variants (CVTT*) mean 'truncate' and ignore the MXCSR rounding mode entirely. Single-T variants respect MXCSR.

| Opcode | Full name | Rounding | Notes |
| --- | --- | --- | --- |
| CVTSS2SI | Convert float32 → int32 | From MXCSR.RC | Respects rounding mode |
| CVTTSS2SI | Convert float32 → int32 | Truncate toward zero | CVTT = always truncate |
| CVTSD2SI | Convert float64 → int32 | From MXCSR.RC | Respects rounding mode |
| CVTTSD2SI | Convert float64 → int32 | Truncate toward zero | Most common in C casts |
| CVTPS2PD | Convert float32×4 → float64×2 | Exact (widening) | No rounding needed |
| CVTPD2PS | Convert float64×2 → float32×4 | From MXCSR.RC | Narrowing — loses mantissa bits |
| VCVTPS2PH | Convert float32 → float16 (AVX) | Immediate bits [2:1] | Per-instruction mode |

## 2.2  MXCSR Rounding Control

The MXCSR register holds SSE state including the Rounding Control field (bits 14:13). Unlike ARM's per-register approach, Intel uses a global MXCSR that applies to all SSE/AVX operations until explicitly changed.

| Bit 14 | Bit 13 | Mode | Equivalent IEEE |
| --- | --- | --- | --- |
| 0 | 0 | Round to Nearest Even (default) | RNE |
| 0 | 1 | Round Toward Negative Infinity | RDN |
| 1 | 0 | Round Toward Positive Infinity | RUP |
| 1 | 1 | Round Toward Zero (truncate) | RTZ |

Intel's x87 FPU has a separate FCW (Floating Point Control Word) with the same four modes but different bit positions. The original x87 default precision was 80-bit extended, which caused the infamous 'double rounding' bugs when results were narrowed to 64-bit IEEE 754 for storage. SSE2 eliminated x87 for most code.

## 2.3  AVX-512 Per-Instruction Rounding

AVX-512 introduced embedded rounding — a 2-bit field in the instruction encoding itself overrides MXCSR for that specific instruction. This allows mixing rounding modes in a single function without saving and restoring the global register.

| {er} bits | Mode | Symbol | Example |
| --- | --- | --- | --- |
| 00 | Round to Nearest Even | {rn-sae} | VCVTSS2SI rax,xmm0,{rn-sae} |
| 01 | Round Down | {rd-sae} | VCVTSS2SI rax,xmm0,{rd-sae} |
| 10 | Round Up | {ru-sae} | VCVTSS2SI rax,xmm0,{ru-sae} |
| 11 | Round Toward Zero | {rz-sae} | VCVTSS2SI rax,xmm0,{rz-sae} |

# 3.  ARM  (AArch64 / A64)

## 3.1  FCVT — Floating-Point Convert

ARM's FCVT (Floating-Point ConVerT) family distinguishes between converting to a different float precision and converting to an integer. The naming encodes direction and rounding mode directly in the mnemonic, which makes ARM assembly more readable than Intel's separate MXCSR approach.

| Opcode | Conversion | Rounding | Notes |
| --- | --- | --- | --- |
| FCVT | fp → different fp precision | Default (from FPCR) | e.g. single to half, double to single |
| FCVTZS | fp → signed integer | Toward Zero (truncate) | Z = toward Zero, S = Signed |
| FCVTZU | fp → unsigned integer | Toward Zero (truncate) | Z = toward Zero, U = Unsigned |
| FCVTMS | fp → signed integer | Toward Minus infinity | M = Minus = floor |
| FCVTMU | fp → unsigned integer | Toward Minus infinity | Floor for unsigned |
| FCVTPS | fp → signed integer | Toward Plus infinity | P = Plus = ceiling |
| FCVTPU | fp → unsigned integer | Toward Plus infinity | Ceiling for unsigned |
| FCVTNS | fp → signed integer | To Nearest (ties to even) | N = Nearest = RNE |
| FCVTNU | fp → unsigned integer | To Nearest (ties to even) | IEEE default behavior |
| FCVTAS | fp → signed integer | To Nearest Away from zero | A = Away = RMM |
| FCVTAU | fp → unsigned integer | To Nearest Away from zero | 'Schoolbook' rounding |

## 3.2  FPCR — Floating-Point Control Register

The FPCR is a per-thread register (not global to the core) that holds the default rounding mode and exception enable bits. ARM's choice to make it per-thread rather than per-core means context switches automatically save and restore the rounding mode, which is why ARM's floating-point behavior is more predictable in multi-threaded code.

| FPCR[23:22] | RMode | Mode | Notes |
| --- | --- | --- | --- |
| 00 | RN | Round to Nearest, ties to Even | IEEE default; most common |
| 01 | RP | Round toward Positive infinity | Ceiling / RUP |
| 10 | RM | Round toward Minus infinity | Floor / RDN |
| 11 | RZ | Round toward Zero | Truncate / RTZ; C cast behavior |

ARM does not implement IEEE 754's RMM mode (round-to-nearest-max-magnitude) as a global FPCR mode — it is only available through the FCVTAS/FCVTAU per-instruction form. This is intentional: RMM is rarely needed globally and its per-instruction form is sufficient.

## 3.3  Sign and Mantissa Handling

ARM AArch64 preserves the sign of NaN inputs when converting between float precisions. When narrowing (double → single, single → half), ARM rounds the mantissa according to the active rounding mode. The exponent is re-biased: single has exponent bias 127, half has bias 15, double has bias 1023. ARM handles the subnormal range explicitly — a value too small for the target format but representable as subnormal is converted to subnormal rather than flushed to zero, unless the FZ (Flush-to-Zero) bit is set in FPCR.

# 4.  RISC-V  (RV64GC / F, D, Q Extensions)

## 4.1  FCVT Family

RISC-V's floating-point conversion opcodes follow a regular naming scheme: FCVT.dest_type.src_type. The rounding mode is encoded in a 3-bit rm field in every floating-point instruction, which can either specify an explicit mode or read from the frm CSR (using rm=111, the DYN mode).

| Opcode | rm field | Conversion | Notes |
| --- | --- | --- | --- |
| FCVT.W.S | rm[2:0] | float32 → int32 | W=word (32-bit signed) |
| FCVT.WU.S | rm[2:0] | float32 → uint32 | WU=word unsigned |
| FCVT.L.S | rm[2:0] | float32 → int64 | L=long (64-bit signed, RV64 only) |
| FCVT.S.W | rm[2:0] | int32 → float32 | Integer to float (may round) |
| FCVT.D.S | rm[2:0] | float32 → float64 | Widening: exact, rm ignored |
| FCVT.S.D | rm[2:0] | float64 → float32 | Narrowing: uses rm field |
| FCVT.Q.D | rm[2:0] | float64 → float128 (Q ext) | Exact for finite values |

## 4.2  rm Field — Per-Instruction Rounding

Every RISC-V floating-point instruction carries a 3-bit rm field. This is a significant design difference from both Intel (global MXCSR) and ARM (per-thread FPCR as default). In RISC-V, the rounding mode is explicitly part of each instruction and the compiler must encode the right mode — it cannot be changed at runtime without changing the instruction stream.

| rm[2:0] | Symbol | Mode | C equivalent | Notes |
| --- | --- | --- | --- | --- |
| 000 | RNE | Round to Nearest, ties Even | Default (fesetround) | IEEE default |
| 001 | RTZ | Round Toward Zero | (int) cast | Truncate |
| 010 | RDN | Round Down (floor) | floor() | Toward -∞ |
| 011 | RUP | Round Up (ceiling) | ceil() | Toward +∞ |
| 100 | RMM | Round to Nearest, Max Magnitude | Custom | Ties away from zero |
| 101 | — | Reserved | — | Invalid — raises exception |
| 110 | — | Reserved | — | Invalid — raises exception |
| 111 | DYN | Dynamic — read from frm CSR | fesetround() | Runtime-configurable mode |

RISC-V's reserved rm values (101, 110) cause an Invalid Operation exception rather than silently using a default. This forces the compiler to always emit an explicit rounding mode, which is stricter than ARM or Intel but prevents the silent wrong-rounding bugs common in cross-platform code.

# 5.  IBM Power / PowerPC

## 5.1  FCTIW / FCTIWZ / FCTID Family

IBM Power uses a different naming philosophy: the opcode itself encodes whether to use the FPSCR rounding mode or to force truncation. Z-suffixed variants always truncate. Power has historically been used in high-precision scientific computing (Cray, early supercomputers) and therefore has the most complete implementation of IEEE 754 modes including full signaling NaN support.

| Opcode | Conversion | Rounding | Notes |
| --- | --- | --- | --- |
| FCTIW | float → int32 | From FPSCR[RN] | Uses global FPSCR register |
| FCTIWZ | float → int32 | Truncate (force RTZ) | Z = toward Zero, ignores FPSCR |
| FCTID | float → int64 | From FPSCR[RN] | 64-bit integer target |
| FCTIDZ | float → int64 | Truncate (force RTZ) | Z suffix = always truncate |
| FRSP | float64 → float32 | From FPSCR[RN] | Round to Single Precision |

## 5.2  FPSCR Rounding Mode

Power's Floating-Point Status and Control Register (FPSCR) bits [31:30] control rounding. IBM is notable for implementing a fifth effective mode: Nearest-Round-Half-Away-From-Zero is accessible via software manipulation of FPSCR rather than a dedicated hardware mode, reflecting Power's heritage in applications where accounting-style rounding matters.

| FPSCR[31:30] | Bits | Mode | Notes |
| --- | --- | --- | --- |
| 00 | RN | Nearest (ties to even) | IEEE default |
| 01 | RZ | Toward Zero (truncate) | C int cast behavior |
| 10 | RP | Toward +∞ (ceiling) | Matches RISC-V RUP |
| 11 | RM | Toward -∞ (floor) | Matches RISC-V RDN |

# 6.  Ternary ISA — Current CVT Opcode

## 6.1  CVT as Defined in ternary_isa.h

The ternary ISA's CVT opcode converts a value from one ternary precision to another. Unlike binary floating-point where format conversion changes bit width, ternary format conversion changes the number of trits — the information density changes in base-3 steps rather than base-2.

| Ternary format | Trits | Binary encoding | Approx. decimal range |
| --- | --- | --- | --- |
| T1 | 1 trit | 8 bits (2 used) | {-1, 0, +1} |
| T5 | 5 trits | 8 bits (10 used) | -121 to +121 |
| T10 | 10 trits | 16 bits | ~±29,524 |
| T20 | 20 trits | 32 bits | ~±1.7 billion |
| T40 | 40 trits | 64 bits | ~±10^19 |
| T50 | 50 trits | 128 bits | ~±10^24 |

## 6.2  CVT Instruction Encoding

CVT is an R-type instruction. The source and destination widths are encoded in the rs2 and func fields respectively. A single CVT instruction handles all combinations: widening (T10 → T50), narrowing (T50 → T10), and cross-family (numeric T20 ↔ lane L20).

CVT.src_width.dst_width  rd, rs1

Example: CVT.t50.t20  r3, r1   ; narrow T50 value in r1 to T20, store in r3

Example: CVT.t10.t50  r5, r2   ; widen T10 value in r2 to T50, store in r5

Example: CVT.t20.l20  r4, r3   ; convert numeric T20 to lane L20 (same trit count)

## 6.3  Current Rounding Behavior

The current CVT implementation in ternary_native_ops.h uses the convertFloat template function which calls packNormalized. When narrowing precision (more trits → fewer trits), the extra mantissa trits are handled by roundedDiv3 — a balanced division that rounds to the nearest representable value by checking whether the remainder is 2 (i.e., the discarded portion was more than half of a trit unit):

// From ternary_native_ops.h — current narrowing behavior:

UInt128 roundedDiv3(UInt128 n) {

uint32_t rem = 0;

UInt128 quotient = n.divMod3(rem);

if (rem == 2) quotient += UInt128{1};  // round up if rem >= 2/3

return quotient;

}

This is equivalent to round-half-up — when the discarded portion is exactly at the boundary (rem == 2 in base 3, meaning the value is exactly halfway between two representable trits), the value is rounded away from zero. This is IBM-style rounding, not IEEE round-to-nearest-even.

The current implementation has no user-controllable rounding mode. The rounding behavior is fixed in the implementation and cannot be changed by the programmer. The proposal in Section 7 changes this.

# 7.  Proposed Ternary-Native Rounding  (Trit-Zone Model)

Background: the binary midpoint problem. Binary floating-point rounding has a fundamental difficulty: the midpoint case. When a value is exactly halfway between two representable values (e.g. 2.5 between 2 and 3), IEEE 754 requires detecting this case and applying a tie-breaking rule (round to even). This requires hardware to determine whether the discarded bits form exactly 0.5, which adds a comparison step and is the source of the 'banker's rounding' confusion in financial software.

Ternary's natural advantage. In balanced ternary, when dropping N trits of precision, the discarded portion maps to one of 3^N possible states. These states divide naturally into thirds: one third rounds down, one third is exact (no rounding needed), and one third rounds up. There is no midpoint to disambiguate. The rounding decision is unambiguous for every possible fractional value.

## 7.1  The 9-Zone Model  (two fractional trits)

When two trits of precision are discarded during a narrowing conversion, those two trits encode 9 possible states in balanced ternary (3^2 = 9). The 9 states map cleanly into three equal groups of three, giving a rounding rule with no special cases.

Encoding convention: the 9 states of two balanced ternary trits range from −4 to +4. Numbering them 1 through 9 (zone 1 = most negative, zone 9 = most positive):

| Zone | Trit₁ | Trit₀ | 2-trit value | Fraction of LST | Rounding action | Analogy |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 | -1 | -4 | −4/9 of LST | Round Down ↓ | Deep negative |
| 2 | -1 | 0 | -3 | −3/9 of LST | Round Down ↓ | Negative |
| 3 | -1 | 1 | -2 | −2/9 of LST | Round Down ↓ | Mildly negative |
| 4 | 0 | -1 | -1 | −1/9 of LST | No change = | Near zero |
| 5 | 0 | 0 | 0 | Exact | No change = | Exact |
| 6 | 0 | 1 | +1 | +1/9 of LST | No change = | Near zero |
| 7 | 1 | -1 | +2 | +2/9 of LST | Round Up ↑ | Mildly positive |
| 8 | 1 | 0 | +3 | +3/9 of LST | Round Up ↑ | Positive |
| 9 | 1 | 1 | +4 | +4/9 of LST | Round Up ↑ | Deep positive |

LST = Least Significant Trit (the smallest representable unit in the target precision).

## 7.2  Why This Is Better Than IEEE Round-to-Nearest-Even

| Property | IEEE RNE (binary) | Trit-Zone (ternary) |
| --- | --- | --- |
| Midpoint case | Special case — needs tie-breaking rule | Does not exist — zones divide evenly |
| Hardware complexity | Comparator + tie-break logic | Zone check: compare 2-trit value to ±1 |
| Statistical bias | None for random data (even/odd balanced) | None — three equal zones |
| Programmer surprise | High — 2.5 rounds to 2, 3.5 rounds to 4 | None — zones are intuitive |
| Symmetric around zero | Yes | Yes — zones are symmetric |
| Representable with trits | N/A | Zone encoded as 2-trit value directly |

## 7.3  Implementation in CVT

The trit-zone model is implemented by examining the two dropped trits when narrowing precision. The decision is a simple comparison against ±2:

// Proposed CVT narrowing with trit-zone rounding:

TernaryValue cvtNarrow(TernaryValue src, TernaryMode target) {

// Extract the mantissa and scale down

Int128 mantissa = mantissaToInt(src.unpack());

Int128 quotient, remainder;

quotient  = mantissa / pow3(dropped_trits);   // integer division

remainder = mantissa % pow3(dropped_trits);   // dropped portion

// Trit-zone decision — remainder maps to 9 zones via 2-trit value

Int128 zone_value = remainder;  // already in balanced ternary range

if      (zone_value <= -2) quotient -= 1;  // zones 1,2,3 → round down

else if (zone_value >=  2) quotient += 1;  // zones 7,8,9 → round up

// zones 4,5,6 (−1 ≤ remainder ≤ +1): no change

return packNormalized(target, quotient, exponent);

}

## 7.4  Generalisation to N dropped trits

For N dropped trits, the remainder has 3^N possible values. The trit-zone model generalises: the lower third of the remainder range rounds down, the middle third produces no change, and the upper third rounds up. The boundary between zones is at ±(3^N - 1)/6, which is always a clean fraction in balanced ternary.

| Dropped trits | States (3^N) | Round down if remainder ≤ | Round up if remainder ≥ | No-change zone |
| --- | --- | --- | --- | --- |
| 1 | 3 | −1 | +1 | {0} |
| 2 | 9 | −2 | +2 | {-1,0,+1} |
| 3 | 27 | −5 | +5 | {-4..+4} |
| 4 | 81 | −14 | +14 | {-13..+13} |
| N | 3^N | ≤ −(3^N−1)/6 − 1 | ≥ +(3^N−1)/6 + 1 | middle third |

For 1 dropped trit, the no-change zone is only {0} — exact. Zones 1 (trit = -1) rounds down and zone 3 (trit = +1) rounds up. This gives the same result as round-half-away-from-zero but with no ambiguity. For 2 dropped trits, the no-change zone is {-1, 0, +1} — values within ±1/9 of an LST are not rounded, which eliminates the accumulation of small rounding errors that plagues floating-point accumulation.

## 7.5  Adding Rounding Mode Control to CVT

To support all rounding modes (for compatibility with IEEE 754 and for precise numerical computation), the CVT instruction encoding can be extended with a 2-trit rounding mode field using the existing pad bits in the R-type instruction word. The ternary ISA has 10 pad trits in R-type — 2 trits suffice for 9 mode combinations, more than binary's 4 modes.

| Mode trit₁ | Mode trit₀ | Symbol | Behavior |
| --- | --- | --- | --- |
| -1 | -1 | RTZ | Round Toward Zero — always truncate toward zero (IEEE RTZ, C int cast) |
| -1 | 0 | RDN | Round Down — toward −∞ (IEEE RDN, floor()) |
| -1 | 1 | RUP | Round Up — toward +∞ (IEEE RUP, ceil()) |
| 0 | -1 | RNE | Round to Nearest Even — IEEE default, banker's rounding |
| 0 | 0 | TZR | Trit-Zone Rounding — proposed native mode: zones 1-3 down, 4-6 exact, 7-9 up |
| 0 | 1 | RMM | Round to Nearest Max Magnitude — ties away from zero |
| 1 | -1 | RAS | Round Away from Sign — toward +∞ for positive values, −∞ for negative values |
| 1 | 0 | DYN | Dynamic — read from TPCR (Ternary Processor Control Register), analogous to RISC-V frm CSR |
| 1 | 1 | — | Reserved — raises TRAP_ILLEGAL_OP |

TZR (Trit-Zone Rounding) is the recommended default for the ternary ISA. It should be the encoding for mode bits (0,0) and should be what the compiler emits when no explicit rounding mode is specified. IEEE compatibility modes (RNE, RTZ, RDN, RUP, RMM) are provided for porting existing numerical code.

## 7.6  The No-Midpoint Proof

The key property that makes TZR superior to RNE can be stated precisely:

In balanced ternary with N dropped trits, there is no value x such that x is equidistant between two adjacent representable values.

Proof sketch: the representable values in the target precision are spaced by exactly one Least Significant Trit (LST). A value halfway between two LSTs would require a remainder of exactly 3^N / 2. Since 3^N is always odd (3^N = 1, 3, 9, 27, ...), it is never divisible by 2. Therefore 3^N / 2 is never an integer, and no binary-style 'exact midpoint' can exist in balanced ternary. The zone boundaries at ±(3^N + 1)/3 are always integers, and every possible remainder falls strictly within one zone — there are no boundary cases.

This proof means TZR is a strictly deterministic rounding function with no special cases, no tie-breaking rules, and no possibility of the accumulation-bias observed in binary RNE rounding when the same value is rounded repeatedly.

# 8.  Cross-Architecture Comparison

## 8.1  Rounding mode summary

| Mode | Intel x86 | ARM AArch64 | RISC-V | IBM Power | Ternary (current) | Ternary (proposed) | IEEE 754? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RNE (nearest even) | MXCSR | FPCR RN | rm=000 | FPSCR RN | Partial | Mode (0,-1) | Yes |
| RTZ (toward zero) | MXCSR / CVTT | FCVTZS | rm=001 | FPSCR RZ | Default | Mode (-1,-1) | Yes |
| RDN (floor, -∞) | MXCSR | FCVTMS | rm=010 | FPSCR RM | No | Mode (-1,0) | Yes |
| RUP (ceil, +∞) | MXCSR | FCVTPS | rm=011 | FPSCR RP | No | Mode (-1,1) | Yes |
| RMM (nearest, away) | No | FCVTAS | rm=100 | Via software | No | Mode (0,1) | Yes |
| TZR (trit-zone) | No | No | No | No | No | Mode (0,0) | Ternary-native |

TZR (Trit-Zone Rounding) is the only mode in this table that is unique to ternary hardware. All other modes are provided for IEEE 754 compatibility. The compiler should default to TZR for ternary-native code and fall back to RNE only when targeting IEEE-compatible output.

Ternary Computer Architecture Project  ·  CVT & Rounding Reference  ·  TZR is the native ternary rounding mode — no midpoint case exists in balanced ternary