Ternary Logic Gates

Complete Reference for Balanced Ternary {−1, 0, +1}

Ternary Computer Architecture Project

# 1.  Balanced Ternary Fundamentals

Balanced ternary uses three digit values: −1, 0, and +1. This is distinct from unbalanced ternary (0, 1, 2) used in some historical computers. The balanced system is preferred for arithmetic because it is symmetric around zero — there is no need for a separate sign bit, negation is exact (flip all trits), and the rounding error for any value is at most half the weight of the least significant trit.

| Trit Value | Symbol | Meaning |
| --- | --- | --- |
| −1 | T or − | Negative / False / Low |
| 0 | 0 | Unknown / Neither / High-Z |
| 1 | 1 or + | Positive / True / High |

Throughout this document: colour-coded trit values use red for −1, grey for 0, and green for +1.

Gate taxonomy. Ternary gates fall into four families: (1) Unary gates — one input, one output; (2) Lattice gates — min/max operations that form a Heyting algebra; (3) Arithmetic gates — addition, multiplication, and modular sum; (4) Comparison and predicate gates.

# 2.  Unary Gates  (single input)

## 2.1  NOT — Negation  (trit flip)

The fundamental unary operation in balanced ternary. Negation simply flips the sign of the trit. Unlike binary NOT which produces a complement, ternary negation is its own inverse: NOT(NOT(a)) = a for all a. In hardware, this requires no logic — it is a wire swap between the two lines of a dual-rail encoding.

Definition:  NOT(a)  =  −a

Corresponding ISA opcode:  NEG  /  TINV

| a | NOT(a) |
| --- | --- |
| −1 | 1 |
| 0 | 0 |
| 1 | −1 |

## 2.2  ABS — Absolute Value

Produces the magnitude of a trit. In the single-trit case this maps −1 → 1, 0 → 0, 1 → 1. Over multi-trit words it is implemented as: if the value is negative, negate it, otherwise pass it through. Equivalent to MAX(a, NOT(a)).

Definition:  ABS(a)  =  MAX(a, −a)

Corresponding ISA opcode:  ABS

| a | ABS(a) |
| --- | --- |
| −1 | 1 |
| 0 | 0 |
| 1 | 1 |

## 2.3  SIGN — Sign Extraction

Extracts only the sign of a value, discarding magnitude. Returns −1 for any negative trit, 0 for zero, +1 for any positive trit. Over multi-trit words it collapses the entire number to a single trit indicating its sign. This is the basis for conditional branching in the ternary ISA.

Definition:  SIGN(a)  =  −1 if a<0,  0 if a=0,  +1 if a>0

Corresponding ISA opcode:  TCMP  (compare against zero)

| a | SIGN(a) |
| --- | --- |
| −1 | −1 |
| 0 | 0 |
| 1 | 1 |

## 2.4  SUCC and PRED — Successor and Predecessor

SUCC increments a trit by one, clamping at +1. PRED decrements a trit by one, clamping at −1. These are useful for counting and for stepping through states in a finite state machine without needing a full adder.

Definition:  SUCC(a) = clamp(a + 1)    PRED(a) = clamp(a − 1)

| a | SUCC(a) | PRED(a) |
| --- | --- | --- |
| −1 | 0 | −1 |
| 0 | 1 | −1 |
| 1 | 1 | 0 |

## 2.5  Predicate Gates — ISPOS, ISNEG, ISZERO

Predicate gates test a single condition and return 1 (true) or 0 (false). They are the ternary equivalents of binary flag checks and form the basis of conditional execution without requiring a compare-and-branch pair.

| a | ISPOS(a) | ISNEG(a) | ISZERO(a) |
| --- | --- | --- | --- |
| −1 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 |

Note: ISPOS(a) is equivalent to MAX(SIGN(a), 0). ISNEG(a) is equivalent to ABS(MIN(SIGN(a), 0)). These identities allow predicate gates to be synthesized from the lattice gates described in Section 3.

# 3.  Lattice Gates  (binary, order-based)

Lattice gates operate on the natural ordering −1 < 0 < +1. They form a distributive lattice and correspond to the Łukasiewicz three-valued logic system. MIN is the lattice meet (conjunction) and MAX is the lattice join (disjunction). Unlike classical binary AND/OR, ternary MIN/MAX are continuous — they degrade gracefully through the intermediate state 0 rather than collapsing to a two-valued result.

## 3.1  MIN — Lattice AND  (meet)

Returns the smaller of two trit values. Generalizes binary AND: when inputs are restricted to {0, 1}, MIN(a, b) equals AND(a, b). The intermediate value 0 acts as a weak false — MIN(0, 1) = 0 rather than jumping directly to −1, which allows gradual degradation in multi-valued logic circuits.

Definition:  MIN(a, b)  =  a  if a ≤ b,  else  b

Corresponding ISA opcode:  TMIN  /  TLAND

| a | b | MIN(a,b) |
| --- | --- | --- |
| −1 | −1 | −1 |
| −1 | 0 | −1 |
| −1 | 1 | −1 |
| 0 | −1 | −1 |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | −1 | −1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

## 3.2  MAX — Lattice OR  (join)

Returns the larger of two trit values. Generalizes binary OR: when inputs are restricted to {0, 1}, MAX(a, b) equals OR(a, b). The value 0 acts as a weak true — MAX(0, −1) = 0 rather than collapsing to +1, preserving the three-way distinction.

Definition:  MAX(a, b)  =  a  if a ≥ b,  else  b

Corresponding ISA opcode:  TMAX  /  TLOR

| a | b | MAX(a,b) |
| --- | --- | --- |
| −1 | −1 | −1 |
| −1 | 0 | 0 |
| −1 | 1 | 1 |
| 0 | −1 | 0 |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | −1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Duality: NOT(MIN(a, b)) = MAX(NOT(a), NOT(b))  and  NOT(MAX(a, b)) = MIN(NOT(a), NOT(b)). De Morgan's laws hold exactly in balanced ternary lattice logic.

## 3.3  NMIN and NMAX — Negated Lattice Gates

NMIN is the ternary equivalent of binary NAND — it negates the MIN result. NMAX is the ternary equivalent of NOR — it negates the MAX result. Both are functionally complete on their own: any ternary function can be expressed using only NMIN gates, mirroring the universality of binary NAND.

Definitions:  NMIN(a,b) = NOT(MIN(a,b))     NMAX(a,b) = NOT(MAX(a,b))

| a | b | NMIN(a,b) | NMAX(a,b) |
| --- | --- | --- | --- |
| −1 | −1 | 1 | 1 |
| −1 | 0 | 1 | 0 |
| −1 | 1 | 1 | −1 |
| 0 | −1 | 1 | 0 |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | −1 |
| 1 | −1 | 1 | −1 |
| 1 | 0 | 0 | −1 |
| 1 | 1 | −1 | −1 |

# 4.  Arithmetic Gates

## 4.1  ADD — Clamped Addition

Single-trit addition with saturation at the extremes. When the true sum exceeds +1 or falls below −1, the result is clamped. In a multi-trit adder, overflow from one trit position generates a carry of +1 or −1 into the next position — the clamped single-trit table below shows the sum without carry. The full carry table is given after.

Definition:  ADD(a,b) = clamp(a + b)   where clamp(n) = max(−1, min(+1, n))

Corresponding ISA opcode:  ADD

| a | b | ADD(a,b) |
| --- | --- | --- |
| −1 | −1 | −1 |
| −1 | 0 | −1 |
| −1 | 1 | 0 |
| 0 | −1 | −1 |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | −1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Full adder: when a = +1 and b = +1, the true sum is +2. In a balanced ternary full adder the output trit is −1 (since +2 ≡ −1 mod 3) and the carry-out is +1. When a = −1 and b = −1, the output trit is +1 and the carry-out is −1.

### Full Adder — Sum and Carry-out

A ternary full adder takes three inputs: two operand trits (a, b) and a carry-in (cin). It produces a sum trit and a carry-out. The table below shows all combinations with cin = 0. The carry-out is always in {−1, 0, +1}.

| a | b | cin | Sum | Carry-out |
| --- | --- | --- | --- | --- |
| −1 | −1 | 0 | 1 | −1 |
| −1 | 0 | 0 | −1 | 0 |
| −1 | 1 | 0 | 0 | 0 |
| 0 | −1 | 0 | −1 | 0 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 1 | −1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | −1 | 1 |

## 4.2  MUL — Ternary Multiplication

Single-trit multiplication. This is the most computationally elegant gate in ternary: the product of two trits is always in {−1, 0, +1} with no overflow possible, because the maximum product is (+1)×(+1) = +1 and the minimum is (−1)×(−1) = +1. Multiplication by 0 always gives 0. Multiplication by −1 is equivalent to negation. This means that in a ternary neural network with weights in {−1, 0, +1}, every weight multiplication is either a pass-through, a negation, or a zero — no actual multiplication hardware is needed.

Definition:  MUL(a, b)  =  a × b  (always in {−1, 0, +1})

Corresponding ISA opcode:  MUL

| a | b | MUL(a,b) |
| --- | --- | --- |
| −1 | −1 | 1 |
| −1 | 0 | 0 |
| −1 | 1 | −1 |
| 0 | −1 | 0 |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | −1 | −1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Hardware insight: a ternary multiplier for {−1,0,+1} operands requires no multiplication logic. It is implemented as: if either input is 0, output 0; if inputs have the same sign, output +1; if inputs have opposite signs, output −1. This is three comparisons and a mux — far cheaper than a binary multiplier.

## 4.3  XSUM — Modular Sum  (trit XOR equivalent)

The modular sum computes (a + b) mod 3, mapping the result back into balanced ternary. This is the direct analog of binary XOR. It is the sum trit of a half-adder (ignoring carry). The operation forms a cyclic group of order 3, which means applying XSUM with a fixed value cycles through all three trit states.

Definition:  XSUM(a,b) = balanced((a + b + 3) mod 3)  mapping: 0→0, 1→+1, 2→−1

| a | b | XSUM(a,b) |
| --- | --- | --- |
| −1 | −1 | 1 |
| −1 | 0 | −1 |
| −1 | 1 | 0 |
| 0 | −1 | −1 |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | −1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | −1 |

# 5.  Comparison and Relational Gates

## 5.1  CMP — Three-way Compare

The most powerful comparison gate in ternary. Returns −1 if a < b, 0 if a = b, +1 if a > b. Unlike binary comparison which produces a boolean, ternary comparison produces a three-valued result that directly drives three-way branching. In the ternary ISA this is the TCMP opcode and its output directly controls BRN (branch if negative), BRZ (branch if zero), and BRP (branch if positive).

Definition:  CMP(a,b)  =  SIGN(a − b)

Corresponding ISA opcode:  TCMP

| a | b | CMP(a,b) |
| --- | --- | --- |
| −1 | −1 | 0 |
| −1 | 0 | −1 |
| −1 | 1 | −1 |
| 0 | −1 | 1 |
| 0 | 0 | 0 |
| 0 | 1 | −1 |
| 1 | −1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

This single gate replaces three binary comparisons (a<b, a==b, a>b) with one ternary comparison. It is the reason the ternary ISA can implement three-way branching (less than / equal / greater than) with a single TCMP followed by three conditional branches, rather than the two comparisons binary ISAs require.

## 5.2  EQ — Equality

Returns +1 if the two trits are equal, 0 otherwise. Note that unlike binary XNOR (which returns 1 for equal and 0 for unequal), ternary EQ uses 0 for unequal rather than −1, making it a predicate rather than a symmetric complement. Implementable as: ISZERO(CMP(a, b)).

Definition:  EQ(a,b)  =  1 if a = b,  else 0

| a | b | EQ(a,b) |
| --- | --- | --- |
| −1 | −1 | 1 |
| −1 | 0 | 0 |
| −1 | 1 | 0 |
| 0 | −1 | 0 |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | −1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

## 5.3  TSEL — Ternary Select  (three-way MUX)

The ternary multiplexer selects one of three inputs based on a control trit. This is not a lookup-table gate in the traditional sense but the fundamental switching element of ternary logic. It is the hardware implementation of the three-way branch and is exposed directly in the ISA as the TSEL opcode.

Definition:  TSEL(ctrl, neg_val, zero_val, pos_val)  =

neg_val  if ctrl = −1

zero_val if ctrl =  0

pos_val  if ctrl = +1

| ctrl | Selected input | ISA branch | Condition |
| --- | --- | --- | --- |
| −1 | neg_val | BRN | a < b |
| 0 | zero_val | BRZ | a = b |
| 1 | pos_val | BRP | a > b |

# 6.  Łukasiewicz and Implication Gates

Ternary logic was studied formally by Jan Łukasiewicz in 1920, who defined a three-valued logic system with specific definitions for negation, implication, and conjunction. These gates are the basis of formal ternary logic proofs and are relevant to the design of a ternary OS's permission model and type system.

## 6.1  IMPL — Łukasiewicz Implication

The ternary implication gate. Returns +1 whenever a ≤ b (the implication a → b holds), and returns b when a > b (partial truth — the implication holds to the degree that b is true). This is the Łukasiewicz definition, which ensures that all classical tautologies remain tautologies in three-valued logic.

Definition:  IMPL(a, b)  =  MAX(NOT(a), b)  =  MAX(−a, b)

| a | b | IMPL(a,b) |
| --- | --- | --- |
| −1 | −1 | 1 |
| −1 | 0 | 1 |
| −1 | 1 | 1 |
| 0 | −1 | 0 |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | −1 | −1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Verification: IMPL(−1, x) = +1 for all x (false implies anything). IMPL(x, +1) = +1 for all x (anything implies true). IMPL(+1, −1) = −1 (true does not imply false). These match classical propositional logic exactly.

## 6.2  CONSENSUS — Median / Average Gate

Returns the median of two values. When both inputs agree in sign, returns that sign. When they disagree, returns 0. This gate models agreement in a voting system: two voters who agree produce a definite outcome; two voters who disagree produce an uncertain outcome. It is the basis of majority-logic circuits in ternary.

Definition:  CONS(a,b)  =  +1 if a+b > 0,  −1 if a+b < 0,  0 if a+b = 0

| a | b | CONS(a,b) |
| --- | --- | --- |
| −1 | −1 | −1 |
| −1 | 0 | −1 |
| −1 | 1 | 0 |
| 0 | −1 | −1 |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | −1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

## 6.3  NMUL — Negated Multiply  (ternary NAND equivalent)

Computes NOT(MUL(a, b)). Since MUL always produces a result in {−1, 0, +1}, NMUL is simply the negation of the product. This gate is functionally complete — any ternary function can be expressed as a composition of NMUL gates.

Definition:  NMUL(a, b)  =  NOT(MUL(a, b))  =  −(a × b)

| a | b | NMUL(a,b) |
| --- | --- | --- |
| −1 | −1 | −1 |
| −1 | 0 | 0 |
| −1 | 1 | 1 |
| 0 | −1 | 0 |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | −1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | −1 |

# 7.  Hardware Implementation Notes

## 7.1  Encoding on Binary Hardware

During the FPGA and simulation phases, ternary trits are encoded as 2-bit pairs on binary hardware. The encoding used throughout this project is:

| Trit value | Binary bits | Hex | Notes |
| --- | --- | --- | --- |
| −1 | 00 | 0x0 | negative / false |
| 0 | 01 | 0x1 | zero / unknown |
| 1 | 10 | 0x2 | positive / true |
| INVALID | 11 | 0x3 | trap — illegal trit state |

The 0b11 pattern is reserved as an illegal state and causes TRAP_ILLEGAL_OP when decoded by the VM or FPGA. This allows hardware to detect corruption in the instruction stream.

## 7.2  Gate Cost Comparison (binary vs ternary)

The following table compares the hardware cost of implementing each ternary gate on binary FPGA fabric (in 4-input LUTs) versus the equivalent gate count on a native ternary ASIC (in ternary cells). These are approximate figures based on synthesis experiments and published literature.

| Gate | Binary LUTs (FPGA) | Ternary cells (ASIC) | Ratio |
| --- | --- | --- | --- |
| NOT / NEG | 2 | 1 (wire swap in dual-rail) | 2× |
| MIN (AND) | 4 | 1 | 4× |
| MAX (OR) | 4 | 1 | 4× |
| MUL | 6 | 2 | 3× |
| ADD (1-trit) | 8 | 3 (half adder) | 2.7× |
| CMP | 10 | 2 | 5× |
| TSEL (MUX) | 12 | 1 (native 3:1 mux) | 12× |
| XSUM | 8 | 2 | 4× |

TSEL (the three-way MUX) shows the largest improvement on native ternary hardware. On binary FPGA it requires 12 LUTs to multiplex three paths based on a two-bit-encoded trit. On a native ternary ASIC it is a single three-input, one-output cell. This is why ternary hardware is fundamentally more efficient for ternary workloads.

## 7.3  Functional Completeness

A set of gates is functionally complete if every ternary function can be expressed using only gates from that set. The following sets are each individually complete:

| Gate set | Notes |
| --- | --- |
| {NMIN} | Single gate — mirrors binary NAND universality |
| {NMUL} | Single gate — negated product is sufficient alone |
| {NOT, MIN, SUCC} | Łukasiewicz basis — standard for formal proofs |
| {NOT, MIN, MAX} | Natural basis — isomorphic to Post's lattice |
| {TSEL, constants} | MUX-based basis — maps directly to hardware synthesis |

# 8.  Mapping to the Ternary ISA

The ternary ISA defined in ternary_isa.h exposes the following opcodes that directly correspond to the gates in this document. This table serves as the bridge between the theoretical gate library and the physical instruction set.

| Opcode | Gate | Section | Description |
| --- | --- | --- | --- |
| NEG / TINV | NOT | 2.1 | Negate all trits of a register |
| ABS | ABS | 2.2 | Absolute value |
| TCMP | SIGN / CMP | 2.3 / 5.1 | Sign of register or difference |
| TMIN | MIN | 3.1 | Lane-wise minimum (AND) |
| TMAX | MAX | 3.2 | Lane-wise maximum (OR) |
| TLAND | MIN | 3.1 | Carryless lane AND |
| TLOR | MAX | 3.2 | Carryless lane OR |
| TLNEG | NOT | 2.1 | Per-trit lane negation |
| ADD | ADD | 4.1 | Ternary addition with carry |
| MUL | MUL | 4.2 | Ternary multiplication |
| TSEL | TSEL | 5.3 | Three-way select on T1 condition |
| BRN | TSEL | 5.3 | Branch if trit[0] = −1 |
| BRZ | TSEL | 5.3 | Branch if trit[0] = 0 |
| BRP | TSEL | 5.3 | Branch if trit[0] = +1 |

Ternary Computer Architecture Project  ·  Internal Reference  ·  All gate definitions use balanced ternary {−1, 0, +1}