Triton-27 Hardware Architecture

FPGA/ASIC Strategy · Hardware Designs · HDL Path · CPU Microarchitecture

Balanced Ternary Co-Design Reference  |  Rev 1.0

# 1  FPGA / ASIC Strategy

## 1.1  Reality Checkpoint

Before committing to a design direction it is worth being precise about what is realistically buildable today versus what becomes viable after further fabrication research.

| Layer | Status | Notes |
| --- | --- | --- |
| FPGA prototype (binary fab) | Buildable now | Standard Xilinx / Intel devices. Dual-rail encoding required. See §1.3. |
| Custom ternary ASIC | Research-stage only | No qualified cell library exists. Multi-voltage SRAM is an unsolved fab problem — see §1.2. |
| 35 % routing reduction claim | Not realisable yet | Benefit materialises only on a native ternary process node. Not on binary FPGA or standard CMOS. |

⚠ ASIC Caveat Ternary SRAM cells and three-voltage signalling are active research problems, not solved engineering. Do not plan production silicon timelines around them. The FPGA dual-rail path is the only proven route today.

## 1.2  Why Native Ternary Silicon Is Hard

Two physical challenges make a native ternary ASIC substantially harder than the architectural description implies:

- Multi-voltage noise margins. Binary logic is reliable because a signal only crosses one threshold. With three voltage levels there are two thresholds, each with roughly half the noise margin of a binary rail. Every node in the signal path must be redesigned, and sensitivity to power-supply noise and temperature variation increases significantly. Multi-level flash cells (MLC/TLC NAND) face the same physics and exhibit higher raw error rates than SLC — the analogy is exact.

- No standard cell library. Standard CMOS PDKs ship with thousands of characterised binary cells (AND, OR, FF, etc.). A ternary process requires equivalent multi-threshold or multi-valued FET cells — none exist in a qualified, commercially available PDK. Building one from scratch is a multi-year foundry engagement before a first test tape-out.

- Ternary SRAM is unsolved. The common claim that ternary stores more entropy per cell than binary 6T SRAM is mathematically true but physically unrealised at scale. A ternary cell needs either two transistors with tightly matched analog thresholds (temperature and aging sensitive) or a binary-encoded cell that often costs more area than it saves.

## 1.3  FPGA Prototype Path (Recommended)

For all near-term prototyping, dual-rail binary encoding on a standard FPGA is the correct choice.

| Encoding | Bits / Trit | 27-trit word | Efficiency | Verdict |
| --- | --- | --- | --- | --- |
| Dual-rail (00=0, 01=+1, 10=−1, 11=fault) | 2 | 54 binary bits | 75 % | ✓ Recommended for prototyping |
| Packed (2 trits per 3 bits) | 1.5 | 42 binary bits | 89 % | Better density, more decode logic — use for later optimisation |
| Unary / thermometer | 3 | 81 binary bits | 53 % | Do not use |

Dual-rail wins for the prototype phase because:

- Routing tools understand it without custom synthesis scripts.

- The illegal state 11 gives free error detection — assert a hardware fault interrupt on any 11 combination.

- Ternary NOT (TNOT) is a zero-gate operation: swap the two rails. This is a real advantage in the ALU.

- FPGA fabric is wide enough that the 25 % capacity waste is irrelevant at prototype scale.

## 1.4  Recommended HDL Path

Standard Verilog/VHDL are binary-biased. Writing the full CPU in raw SystemVerilog with manual dual-rail structs is maintainable but verbose. The recommended path:

★ Recommended Use SpinalHDL or Chisel. Define a Trit bundle type ({plus: Bool, minus: Bool}), overload arithmetic operators on Vec[Trit], and add hardware assertions that kill synthesis if any 11 state reaches a register. The HLS compiler generates dual-rail Verilog for the FPGA target automatically. This keeps HDL code at roughly the same abstraction level as TCL 1.0 source — which matters when the same team writes both layers.

If Chisel/SpinalHDL is unavailable, the SystemVerilog struct approach is workable:

// Single trit as a packed struct (SystemVerilog)

typedef struct packed {

logic plus;

logic minus;

} trit_t;

// 27-trit word

typedef trit_t [26:0] tryte27_t;

// TNOT: zero gate cost — just swap rails

function automatic trit_t ternary_not(input trit_t t);

return '{plus: t.minus, minus: t.plus};

endfunction

// Fault assertion — fires in simulation and can drive a fault interrupt in synthesis

assert property (@(posedge clk) !(t.plus && t.minus))

else $error("Illegal dual-rail state 11 detected");

Bluespec is also worth noting: its rule-based atomic execution model maps naturally onto the two-tiered scheduler design (Tier-1 flat run-queue rules vs. Tier-2 reconciliation rules) and onto the WAL transaction semantics in Phase D.

# 2  Hardware Designs

## 2.1  Ternary MMU Page Table Entry (PTE)

A single native 27-trit machine word. Supports Phase D9 COW forks, hardware namespace isolation, and three-level privilege enforcement.

Trit:  26   25   24   23   22   21  | 20 ──── 15 | 14 ──────────────── 0

│    │    │    │    │    │       │              │

Vld  Prv  Ex   Wr   Rd  COW   Namespace ID   Physical Page Number

| Trits | Field | Encoding | Function |
| --- | --- | --- | --- |
| 0 – 14 | PPN | 3^15 = 14,348,907 page addresses | Physical page frame base address. |
| 15 – 20 | Namespace ID | 3^6 = 729 hardware address domains | Hardware-enforced address-space isolation filtering. |
| 21 | COW | neg = writable; pos = copy-on-write active | Triggers hardware page duplication on write-fault. |
| 22 | R | neg = read disabled; pos = read allowed | Page read permission. |
| 23 | W | neg = write disabled; pos = write allowed | Page write permission. |
| 24 | X | neg = non-executable; pos = executable | Prevents user data-page execution. |
| 25 | Privilege | neg = kernel; zero = supervisor; pos = user | Maps directly to PMR execution level. |
| 26 | Valid | neg = fault / unmapped; zero = reserved; pos = present | Triggers page-in or null-pointer trap. |

⚠ Cold-Boot Fix vs. Original Design The original design left the Valid field's zero state undefined. On cold boot DMEM initialises to zero, so every PTE starts in an unspecified state — a potential silent misinterpretation. The revised encoding above explicitly defines zero as 'reserved / transitional', making neg the guaranteed fault state. This must be matched by the MMU's fault-path logic.

## 2.2  Dual-Rail 1-Trit Register Cell (FPGA)

Each balanced-ternary trit maps to two physical binary flip-flops. A hardware AND gate continuously monitors for the illegal 11 state and asserts a fault interrupt.

| Plus rail | Minus rail | Ternary value | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Zero |
| 1 | 0 | +1 | Positive |
| 0 | 1 | −1 | Negative |
| 1 | 1 | — | ILLEGAL — hardware fault interrupt asserted immediately |

┌──────────────────────────────────────────────────┐

│  In+ ──→ D-FF+ ──→ Out+                         │

│              └──────────────────┐               │

│                                 AND ──→ FAULT   │

│              ┌──────────────────┘               │

│  In− ──→ D-FF− ──→ Out−                         │

└──────────────────────────────────────────────────┘

★ Design Note TNOT costs zero gates on this encoding: negation is a rail swap (Plus ↔ Minus). Ensure synthesis tools know this is a structural equivalence, not a new combinational path, or they will waste LUTs.

⚠ Missing From Original Design The fault output was left unconnected — 'System Fault Interrupt' with no further specification. Before synthesis this must be tied to a concrete pipeline action: does it stall the pipeline, trigger a trap to kernel_dispatch, or assert a bus-error register? All three are valid; pick one and document it.

## 2.3  Tier-1 Synchronisation: Hardware Snoop Monitor

Enforces atomicity of the TLDR (Ternary Load-Linked) / TSTR (Ternary Store-Conditional) instruction pair without software polling overhead.

| Step | Event | Hardware Action |
| --- | --- | --- |
| 1 | Core executes TLDR at address A | Load address A into Address Lock Register. Set Lock Valid Flag to +1 (pos). |
| 2 | Any other core or DMA writes to address A | Hardware Address Comparator detects match on the system bus. Clears Lock Valid Flag to −1 (neg) within one clock cycle. |
| 3 | Local core also writes to address A before TSTR | Self-invalidation path: same comparator logic applies to local store traffic. Flag clears to −1. (See note below.) |
| 4 | Core executes TSTR | Reads Lock Valid Flag. pos → write commits, result +1. neg → write aborts, result −1. Retry loop in software. |

⚠ Self-Invalidation Gap in Original Design The original diagram showed only foreign-core invalidation. A local store to the locked address between TLDR and TSTR must also clear the lock — otherwise a local write succeeds when it should abort, silently corrupting the atomicity guarantee. Step 3 above adds this path explicitly.

# 3  Triton-27 CPU Microarchitecture

## 3.1  Core Specifications

| Parameter | Value |
| --- | --- |
| Native word width | 27 trits (≈ 42.8 bits of information entropy) |
| Pipeline depth | 5 stages, single-issue, in-order |
| General-purpose registers | 24 (r1–r24) — matches Phase C3 graph-colouring pool exactly |
| System registers | r25 SP, r26 LR, CR (context ptr), PMR (privilege + namespace) |
| Instruction format | Fixed 27-trit words: R / I / B formats per ISA spec |
| Multiplier | Multi-cycle co-unit (NOT single-cycle) — see §3.3 |

## 3.2  5-Stage Pipeline

Data flows through five stages with the MMU and snoop monitor integrated directly into the pipe. The diagram below shows the structural handshakes.

┌─────────────────────────────────────────────────────────────────┐

│  IF: PC → MMU_IF → I-Cache → IR                                │

│   ↓                                                             │

│  ID: IR → Ternary Decode → Register File  +  Priv/Mode Control │

│   ↓                                                             │

│  EX: Reg A/B → Balanced Ternary ALU ↔ Snoop Monitor            │

│               Priv/Mode Control gates protected instructions    │

│   ↓                                                             │

│  MEM: ALU addr → MMU_MEM (enforces COW + Namespace) → D-Cache  │

│   ↓                                                             │

│  WB: Load data / ALU result → MUX → Register File              │

└─────────────────────────────────────────────────────────────────┘

## 3.3  Functional Units

### Balanced Ternary ALU

Because digits already carry their own sign (−1, 0, +1), the ALU needs no separate signed/unsigned execution tracks and no two's complement inversion path. This genuinely simplifies gate topology and aids pipeline timing closure — one of the few cases where balanced ternary offers a concrete binary-equivalent advantage.

- Addition: carry-save ternary adder matrix. Sum and carry both lie in {−1, 0, +1}.

- Subtraction: identical topology to addition — negate operand B (zero gate cost on dual-rail) then add.

- Comparison (TCMP): produces T1 result directly; feeds branch unit without a second compare cycle.

### Hardware Multiplier — Multi-Cycle Unit

⚠ Critical Correction The original design implied a single-cycle 27×27 trit multiplier in the EX stage. On dual-rail FPGA encoding this is a 54×54 binary multiplier — a large combinational block whose critical path would severely limit the clock frequency. This must be a multi-cycle co-unit with a handshake (ready/valid) signal, not a single-stage operation. Montgomery multiplication (48-limb CIOS) absolutely cannot fit in one pipeline stage; it is a coprocessor with a completion interrupt or a long-latency instruction that issues a pipeline stall for N cycles.

Recommended approach: treat the multiplier as a 3–5 cycle pipelined functional unit for single 27-trit products. For the 48-limb Montgomery loop, expose a dedicated MMIO-mapped coprocessor that the kernel driver sequences, matching the assembleMontgomeryOuterStep reference model.

### Privilege & Mode Control Unit

Inspects every decoded instruction against the PMR before the ALU stage. If a user-space thread (PMR = pos) issues a protected instruction, the unit suppresses EX, raises an Illegal Opcode Exception, and branches to kernel_dispatch at Phase D1's trap vector. The namespace ID field in the PMR is forwarded to the MMU at the MEM stage for PTE namespace filtering.

## 3.4  System Call Sequence

| Step | Action |
| --- | --- |
| 1 SYSCALL issued | User thread places arguments in r1–r3, executes SYSCALL instruction. |
| 2 Privilege transition | Priv/Mode Unit shifts PMR from pos (user) → neg (kernel). Stack pointer switches to kernel stack. Active Namespace ID is latched. |
| 3 Trap dispatch | Execution branches to kernel_dispatch (Phase D1). MMU validates calling addresses against PTE Namespace ID trits. |
| 4 Transaction completes | Kernel executes ERET. PMR restored to user. Process-local cache invalidated via FENCE.0 if required. Control returns to user thread. |

# 4  Cryptographic Hardware Integration

## 4.1  RSA-2048 Montgomery vs. BLAKE3 — Role Separation

These two algorithms serve complementary, non-overlapping roles. Neither replaces the other.

| Dimension | Ternary Montgomery / RSA-2048 | Ternary BLAKE3 (planned) |
| --- | --- | --- |
| Category | Asymmetric public-key (signatures, key exchange) | Symmetric hash / MAC / KDF / XOF |
| Math basis | Big-integer modular exponentiation, O(N²) limb multiply, base 3^27 | ARX rounds (add, rotate, XOR), O(N) linear, 32-bit words |
| Parallelism | Sequential Montgomery ladder — constant time, not parallelisable | Unbounded: Merkle tree over 1 KiB independent chunks |
| OS role | Secure boot, executable signature verify, POST self-test | Block checksum, data deduplication, key derivation, fast file hashing |
| Hardware unit | Dedicated MMIO coprocessor (see §3.3) | SIMD lanes in ALU or separate hash accelerator |

## 4.2  Smart Card MMU Gate

The RSA coprocessor pages are mapped with Execute-only permission (X=pos, R=neg, W=neg) in the PTE. DMEM[0] acts as the gate word. The kernel POST (Phase D, boot) executes runToySelfTest() against a known N=8633 vector before enabling the coprocessor for user-accessible syscalls. A fault-injection failure (alpha particle, hardware glitch) locks the coprocessor path permanently until reboot.

# 5  Open Items Before Synthesis

- PTE Valid field zero state. Must be explicitly handled in MMU fault-path RTL. Zeroed DMEM on cold boot must not be silently interpreted as 'present'.

- Fault output wiring. Dual-rail cell fault signal must be tied to a concrete pipeline action (stall, trap, bus-error register). Document the choice before synthesis.

- Snoop monitor self-invalidation. Add local-core store path to the Address Comparator. Omitting it creates a silent atomicity bug.

- Multiplier latency model. Decide: pipelined co-unit (3–5 cycles with ready/valid) or stall-on-issue. Write the latency into the pipeline hazard table before RTL.

- HDL framework selection. Chisel or SpinalHDL recommended. If using SystemVerilog, the trit_t struct library and assertion framework must be built first.

- sys_write_char syscall. The VM currently only supports sys_write_int. Without a char-output syscall, compiler error messages output decimal character codes — unreadable. Add before starting tritc.trit debug cycles.

Triton-27 Hardware Architecture  |  Rev 1.0  |  Balanced Ternary Co-Design Reference